INSTRUCTIONS

NOTE: The center divider cannot be glued together until the last step. The 3 screws will not hold the weight of the 4 drawers when they are filled up.

The center divider / bracket is now milled to 1/32nd, so it may still be a tight fit. If it is too tight more will have to be taken off.

To test this, replace the center divider / bracket with the 4 machine screws and tighten down.

Place a drawer (1) into one side toward the bottom and slide it up into the top position.

Do the same to the other side, sliding the drawer (2) up into the top position. The two top drawers should hold each other in for the next step.

The drawer slides should be fairly tight against the sides, enough to gently try the drawer and test for binding.

Regardless of how they work, slide the next drawer (3rd) into the bottom opening and adjust for height front and back.

Replace the last drawer (4)  into the bottom opening and adjust front and back.

With a flashlight make a mark in the front of the case with the drawer slides at the position where they work best, not necessarily in the same place as the original slides.

IF OTHER WORK IS NEEDED:

Bring the center divider / bracket back to be milled down further.

IF NO OTHER WORK IS NEEDED:

After marks are made, remove the drawer slides from the drawers and mount them onto the bench case using the front marks and then level the back end to it.

With the slides in the best position, double check to make sure front to back is level.

Remove the center bracket and remove the 3 screws holding the bracket together.

Glue the bracket to the base but DO NOT get any glue on the dowel pins. Replace the 3 screws and re-assemble the bench.